# Custom Build Brompton — Planning Document

Prepared for Asri · August 2026 · All prices in SGD equivalent, gathered from live listings/forums as of Aug 2026 — treat as indicative, confirm before ordering.

# 1. Requirements Recap

- Front and rear child seat capability required.

- Weight saving is a priority.

- Cost effectiveness is a priority.

- You travel for work — can buy overseas (incl. China) and collect/ship to a hotel.

- All costs quoted in SGD equivalent; tourist GST/tax rebates and Singapore import GST NOT factored in.

# 2. Executive Summary & Recommendation

Two viable paths were evaluated: (A) buy a used genuine Brompton (16" wheel) and modify it with titanium parts for weight savings, or (B) self-build a 20" titanium Brompton-clone from a Chinese frame kit. The table below summarises the trade-off.

| Criteria | Option A: Used Brompton (16") + Ti mod | Option B: 20" Ti clone self-build |
| --- | --- | --- |
| Total cost (parts, excl. tools) | ~S$2,650 – 2,900 | ~S$2,750 – 3,300 |
| Bare-bike weight (both seats removed) | ~10.3 – 10.6 kg | ~8.5 – 9.5 kg |
| Child-seat compatibility | High — genuine Brompton geometry, all Brompton-fit seats/adapters work directly | Medium/Low — clone frame tube diameters & geometry vary by factory batch; brackets often need shimming/custom fit |
| Build difficulty / tools | Moderate — swapping fork, rear triangle, seatpost on a known platform | High — full build from bare frame: wheelbuild, groupset install, brake/gear setup, alignment |
| Risk | Low — well-trodden mod path, resale value retained (genuine Brompton) | Higher — frame quality varies by factory batch, no warranty, IP/trademark grey area, harder resale |
| Fold compatibility (train/plane/hotel storage) | Identical to stock Brompton fold | Usually mimics Brompton fold but tolerances can be looser — verify before buying |

Recommendation: Go with Option A (used 16" Brompton + titanium fork/rear-triangle/seatpost mod) as your primary build.

- It is cheaper, lighter to de-risk (you start from a working, foldable, warrantied-fold-mechanism bike), and every child-seat product on the market is designed around genuine Brompton dimensions — critical since you need both front AND rear seats to fit properly and fold-compatibly.

- Treat Option B (20" titanium clone) as a secondary/future project once you've tested the mod path — build it as a second, ultralight bike for solo commuting once the kids have outgrown seats, not as the child-seat bike.

# 3. Option A — 16" Used Brompton + Titanium Modification

## 3.1 Bill of materials & cost (SGD equivalent)

| Item | Est. cost | Source / notes |
| --- | --- | --- |
| Used Brompton, steel M/A/C-type, 3-speed, good condition | S$650 – 950 | Carousell (SG, no shipping) is cheapest net-of-effort; Gumtree/Preloved UK ~£300-420 (~S$520-730) is cheaper on paper but shipping a full bike overseas is costly/impractical — buy locally or during a UK trip and check it in as luggage |
| Titanium front fork (EasyTi or equivalent) | S$260 – 380 | Taobao direct via forwarding agent cheapest; eBay/EasyTi-brand resellers ~S$460-520 (2-3x Taobao) |
| Titanium rear triangle | S$450 – 650 | Taobao direct cheapest; branded eBay listings ~S$750-800 |
| Titanium seatpost | S$110 – 160 | Taobao/AliExpress |
| Titanium bolt kit (full set, incl. hinge clamp bolts) | S$90 – 130 | Taobao/AliExpress |
| Lightweight tyres (e.g. Marathon Racer / Kojak, pair) | S$80 – 100 | Local bike shop or Taobao |
| Front child seat (mounted to head-tube block, e.g. Bobike Mini/Bromptonic-style) | S$180 – 260 | Taobao direct (S$180-220) vs branded EU resellers (Bromptonic ~S$260, Bobike+adapter ~S$330) |
| Rear child seat + mount (e.g. Milian Pere-style, clamps to seat tube) | S$230 – 330 | Taobao direct (S$230-280) vs Milian/SJS Cycles UK (~S$330) |
| Pentaclip / seat-clamp compatibility hardware (if needed for rear seat) | S$35 – 50 | Brompton spares |
| Consumables (grease, cable, housing, misc.) | S$40 – 60 | Local |

Subtotal (parts): roughly S$2,125 – 3,570, midpoint ≈ S$2,650 – 2,900 once you settle on Taobao-sourced titanium and one seat brand at each end.

## 3.2 Weight plan

| Configuration | Est. weight |
| --- | --- |
| Stock steel Brompton (3-speed, standard steel fork/rear triangle) | ~11.2 – 11.5 kg |
| After: Ti fork (-0.35kg), Ti rear triangle (-0.45kg), Ti seatpost (-0.15kg), lighter tyres (-0.1kg) | ~10.2 – 10.5 kg (bare bike) |
| + Front child seat installed | +1.0 – 1.3 kg |
| + Rear child seat + mount installed | +1.4 – 1.8 kg |
| Both seats fitted, ready to ride | ~12.6 – 13.6 kg total |

Note: child seats are removable/quick-detach on both ends, so day-to-day solo folding weight stays at the ~10.3kg bare-bike figure — most of the weight-saving budget is best spent on the fork and rear triangle since those give the biggest kg-per-dollar return.

## 3.3 Tools required

- Brompton-specific 2/2.5/3/4/5/6mm hex (Allen) key set + Torx T25/T30 (for hinge clamps and some Ti bolt kits)

- Torque wrench (2-20 Nm range) — critical for titanium bolts and hinge clamps, which are easy to over- or under-torque

- Pedal spanner / 15mm cone wrench

- Chain whip + cassette/freewheel removal tool (if swapping drivetrain parts)

- Bottom bracket tool (square-taper or splined, depending on BB type)

- Tyre levers + track pump

- Cable cutters, housing cutter

- Grease (standard + anti-seize/Ti-prep for titanium bolt threads — important to prevent galling)

- Bike repair stand (optional but strongly recommended for fork/rear-triangle swap)

- Estimated tool cost if starting from scratch: S$150 – 260 (many items are one-off/reusable for future bikes)

# 4. Option B — 20" Titanium Brompton-Clone Self-Build

## 4.1 Bill of materials & cost (SGD equivalent)

| Item | Est. cost | Source / notes |
| --- | --- | --- |
| Titanium frame + fork + rear-triangle kit (20" trifold, Brompton-clone geometry) | S$550 – 950 | Taobao direct via agent is cheapest (factories: Hexati/EasyTi/"Comeplay" and similar OEMs); pre-assembled complete bikes from tibicycles.com (Singapore-based, TaiSeng) run S$1,900 – 5,900 all-in, no self-build or shipping needed but 2-4x the DIY parts cost |
| Drivetrain: derailleur, shifter, crankset, BB, chain, cassette/freewheel | S$300 – 450 | Taobao/AliExpress or reuse a donor groupset |
| Wheelset (20" alloy, hand-built or pre-built) | S$180 – 400 | Alloy cheapest; carbon wheelset adds S$300-600+ |
| Brakes (rim or disc, incl. levers) | S$60 – 150 | Disc adds cost & weight vs. rim |
| Handlebar (M/H/P/S-type equivalent), stem, grips | S$60 – 120 |  |
| Saddle + Ti or alloy seatpost | S$70 – 150 |  |
| Folding pedals | S$40 – 90 |  |
| Front child seat + mount (expect custom shimming for clone tube diameter) | S$180 – 260 | + S$50-100 contingency for bracket fitting/fabrication |
| Rear child seat + mount (expect custom shimming) | S$230 – 330 | + S$50-100 contingency |
| Cables, housing, bolts, consumables | S$80 – 120 |  |

Subtotal (parts): roughly S$1,850 – 3,470, realistic midpoint ≈ S$2,750 – 3,300 including fitting contingency for the child-seat brackets.

## 4.2 Weight plan

| Configuration | Est. weight |
| --- | --- |
| Ti frame + fork + rear triangle only | ~1.6 – 2.0 kg |
| Complete build, light alloy wheels, rim brake, 7-9 speed | ~8.5 – 9.5 kg (bare bike) |
| Complete build, carbon wheels, disc brake, higher-spec groupset | ~7.5 – 8.3 kg (bare bike) — but pushes cost toward S$4,000+, working against your cost target |
| + Both child seats fitted | +2.4 – 3.1 kg → ~11 – 12.6 kg total |

## 4.3 Tools required (all of Option A's tools, plus)

- Wheel truing stand + spoke key (or outsource wheel-building to a local shop, ~S$40-60/wheel labour)

- Headset press / headset cup tool (installing headset into a bare frame)

- Crank puller (if not using self-extracting cranks)

- Cable-routing/housing gauge for a frame with unfamiliar internal routing

- Brake bleed kit (if going hydraulic disc)

- Digital caliper (to check clone frame tube diameters before ordering child-seat clamps/adapters)

- Estimated incremental tool cost: S$150 – 300 on top of Option A's kit

# 5. Front & Rear Child Seat Options (either build)

| Product | Position | Est. price (SGD) | Notes |
| --- | --- | --- | --- |
| Genuine-style front seat (Bromptonic-brand, UK-based) | Front, head-tube block mount | ~S$260 | Free worldwide shipping, 12-20 days to Asia; for ages ~2-6, up to 125cm |
| Bobike Mini/One + Milian "Bryan" adapter | Front, handlebar-area mount | ~S$300 – 350 (seat + adapter) | Requires the adapter block; works with most handlebar types |
| Milian "Pere" rear seat + mount | Rear, seat-tube clamp | ~S$330 | Ages ~2.5-5/6; needs a Brompton Pentaclip seat clamp; adds ~3cm protrusion when bike is folded |
| Fantastic4 (F4) quick-detach child seat | Rear (quick-release, foldable) | ~S$80 (925g) | Cheapest branded option found; shipping origin not disclosed on listing — verify lead time before relying on it for a trip |
| Taobao-direct equivalents (search "小布儿童座椅" / "折叠车儿童座椅") | Front or rear | ~S$60 – 150 each | Same factories supply several Western-branded resellers at 2-3x markup; biggest single cost-saving lever in this whole plan — worth sourcing both seats this way |

# 6. Where to Buy Cheapest (incl. China) & Shipping-to-Hotel Logistics

## 6.1 Recommended sourcing by item

- Base used Brompton: Carousell (Singapore) first — zero shipping/customs friction, easy to inspect hinge wear in person. Only consider UK/Japan used listings (Gumtree, Preloved, Mercari, Yahoo! Auctions Japan) if you're already travelling there and can fold it into checked luggage — shipping a full bicycle internationally usually costs more than the savings.

- Titanium frame parts (fork, rear triangle, full frame kits): Taobao direct is consistently the cheapest — often half to a third of the price of the same part relisted on eBay/AliExpress by a Western reseller (e.g. EasyTi-branded parts on eBay run noticeably higher than the same factory's Taobao listings). You will need a China forwarding/agent service (e.g. Superbuy, CSSBuy, Wegobuy, Sifubuy) since Taobao does not ship internationally on its own.

- Pre-built titanium clone complete bikes: tibicycles.com is Singapore-based (TaiSeng MRT) — convenient, no import hassle, but 2-4x the cost of self-sourcing parts. Use this only if you want a turnkey Option B bike and are willing to pay the assembly/convenience premium.

- Child seats: Taobao-direct is again cheapest; Western brands (Bromptonic, Milian, Bobike) are 2-3x the price for what is largely the same manufacturing base.

- Tools: generic hex/torque wrench sets are cheapest on Taobao/AliExpress; buy Brompton-specific torque specs from Brompton's official spares list even if buying cheap tools.

## 6.2 Shipping to an overseas hotel while travelling

- Use a single China forwarding agent to consolidate multiple Taobao purchases (frame, fork, rear triangle, both seats, bolt kit) into one parcel — cuts per-shipment freight fees significantly versus shipping each item separately.

- Confirm with the hotel in advance (email/WhatsApp) that they accept package holding for an in-house guest, and address the parcel exactly as: your full name, "Guest arriving [date], reservation #[xxxx]", hotel name and address, hotel front-desk phone number.

- Build in a buffer: have the agent ship so the parcel arrives 3-5 days before your check-in, and get tracking with signature-on-delivery where possible so it isn't sitting unclaimed at the front desk too long.

- For bulky items (full frame kit, wheels), check the hotel's package size policy — some business hotels cap parcel size/weight they'll hold for guests.

- Carry the assembled/folded bike home in checked luggage or as an oversized item rather than shipping the finished bike — shipping a fully built bike internationally is expensive and risks transit damage; shipping loose parts and assembling at home (or a local shop) is both cheaper and safer.

- As requested, this plan excludes Singapore GST import cost and tourist-scheme tax rebates from all figures above — factor Singapore's prevailing import GST rate separately when the parcels actually land, since it will add to the landed cost.

# 7. Key Risks & Caveats

- IP/trademark: Brompton's original folding-hinge patents have expired, which is why clone frames (from Hexati, EasyTi, Pacific/Pikes-style factories, etc.) are widely available, but the Brompton brand/logo itself is still trademarked — buy "Brompton-compatible"/"trifold" clone frames, not counterfeit-branded ones.

- Clone frame tolerances vary by factory batch — order a digital caliper reading of head-tube and seat-tube diameters from the seller before committing to a child-seat mount purchase, since seat/adapter fit is the single biggest risk in Option B.

- Titanium-to-steel/alloy interfaces need anti-seize/Ti-prep grease to avoid galling; skipping this is the most common DIY mistake reported in the forums researched.

- A modified used Brompton (Option A) retains most of its resale value since it's still a genuine frame; a clone-frame Option B build has little to no resale market — treat it as a build-to-keep, not a build-to-flip.

- Inspect any used Brompton's frame hinge and main hinge clamp for play/cracking before buying — this is the highest-wear part on high-mileage units and the costliest to replace.

# 8. Suggested Next Steps

- 1) Start watching Carousell for a well-maintained 3-speed steel Brompton in your size/colour preference; budget S$650-950.

- 2) In parallel, identify a China forwarding agent and get comfortable with the Taobao purchase flow before your next work trip.

- 3) Order the titanium fork + rear triangle + bolt kit + both child seats as one consolidated Taobao shipment, timed to land at your destination hotel 3-5 days before a planned trip, or simply to your home/office if no trip is imminent.

- 4) Assemble Option A first; live with it for a season to validate both child seats fit your kids and your routine.

- 5) If you still want a second, lighter bike afterward, revisit Option B as a dedicated solo/commuter build once you've got hands-on experience from Option A.